CAS PLATFORM

Collision Avoidance System

Software Requirements Specification

SRS-CAS-001 Rev 1.0

Aligned with ECSS-E-ST-40C

ESA Technology Readiness Level 5 Target

Document Date: March 2026

Classification: UNCLASSIFIED

| Author | Mustafa Firinci / CAS Platform |
| --- | --- |
| Organization | casplatform.com |
| Contact | mustafa@casplatform.com |
| Version | 1.0 (Initial Release) |

# 1. Introduction

## 1.1 Purpose

This Software Requirements Specification (SRS) defines the functional and non-functional requirements for the CAS (Collision Avoidance System) Platform. CAS is a space debris collision avoidance system that monitors tracked objects in orbit, computes collision probabilities in real time, and delivers actionable avoidance maneuver recommendations.

This document is aligned with ECSS-E-ST-40C (Space Engineering — Software) and supports the ESA Technology Readiness Level 5 assessment per ISO 16290.

## 1.2 Scope

CAS Platform encompasses the following subsystems:

- CAS Engine — Python-based orbital mechanics computation backend

- Web Dashboard — HTML/JS operator interface with 3D visualization

- Operator Portal — Authentication, API management, conjunction history

- REST API — Programmatic access for external system integration

- Alert System — SMTP email notifications for RED conjunction events

- Live Catalog — Real-time Celestrak satellite catalog browser

## 1.3 Definitions and Acronyms

| Term | Definition |
| --- | --- |
| CDM | Conjunction Data Message — standardized format for conjunction event data from 18th Space Control Squadron |
| Pc | Collision Probability — probability of two objects colliding at closest approach |
| TCA | Time of Closest Approach — predicted time when two objects reach minimum separation |
| TLE | Two-Line Element set — standardized orbital element format for satellite tracking |
| SSA | Space Situational Awareness — comprehensive knowledge of the space environment |
| SGP4 | Simplified General Perturbations 4 — standard orbital propagation algorithm |
| HBR | Hard Body Radius — combined effective collision cross-section radius (default 10m) |
| JWT | JSON Web Token — compact token format for authentication |

## 1.4 Reference Documents

- ISO 16290:2013 — Space systems: Definition of TRLs and criteria of assessment

- ECSS-E-ST-40C — Space Engineering: Software

- ECSS-Q-ST-80C — Space Product Assurance: Software Product Assurance

- ESSB-HB-E-002 — Guidelines for the use of TRLs in ESA programmes

- Foster/Chan 2D Gaussian Pc method — ESA Space Debris Office methodology

- CCSDS 508.0-B-1 — Conjunction Data Message standard

# 2. System Overview

## 2.1 System Architecture

CAS operates as a monolithic Python HTTP server with PostgreSQL persistence. The system ingests orbital data from two primary sources: Celestrak (TLE data) and Space-Track (CDM data). All computation is performed server-side using the Foster/Chan 2D Gaussian method with J2 perturbation-corrected SGP4 propagation.

## 2.2 Operational Environment

- Server: Ubuntu 24.04 LTS, Python 3.12, PostgreSQL 16

- Hosting: EU-based VPS (GDPR-compliant infrastructure)

- Frontend: HTML5/CSS3/JavaScript, Chart.js 4.4, Three.js r128

- External Data: Celestrak GP API, Space-Track REST API

- Email: SMTP via Namecheap Private Email (TLS 587)

## 2.3 Domain of Applicability

CAS is designed for collision avoidance in the following orbital regimes:

| Regime | Altitude | Status |
| --- | --- | --- |
| LEO | 200– 2,000 km | Primary domain. Validated with Space-Track CDM data. 99.9% Pc accuracy. |
| MEO | 2,000–35,786 km | Supported via TLE propagation. Not yet validated with CDM data. |
| GEO | 35,786 km | Supported via TLE propagation. Celestrak GEO debris catalog integrated. |

# 3. Functional Requirements

## 3.1 Conjunction Detection and Analysis

| ID | Requirement | Priority | Status |
| --- | --- | --- | --- |
| FR-CD-01 | System shall propagate TLE data using SGP4 with J2 perturbation corrections for configurable time horizons (24h to 120h). | MUST | Done |
| FR-CD-02 | System shall detect all conjunction events where miss distance falls below configurable threshold (default: 10 km). | MUST | Done |
| FR-CD-03 | System shall compute collision probability using Foster/Chan 2D Gaussian method with configurable sigma (default: 100m) and HBR (default: 10m). | MUST | Done |
| FR-CD-04 | System shall classify conjunctions into RED (Pc > 1e-4), YELLOW (Pc > 1e-5), and GREEN (Pc < 1e-5) risk categories. | MUST | Done |
| FR-CD-05 | System shall ingest Space-Track CDM data via authenticated REST API with configurable fetch parameters (days, min_pc). | MUST | Done |
| FR-CD-06 | System shall support automated hourly CDM fetch via cron scheduler. | MUST | Done |
| FR-CD-07 | System shall display Pc trend over time for individual conjunction events across multiple CDM updates. | SHOULD | Done |
| FR-CD-08 | System shall compare Space-Track Pc values against CAS engine Pc calculations with side-by-side visualization. | SHOULD | Done |

## 3.2 Avoidance Maneuver Computation

| ID | Requirement | Priority | Status |
| --- | --- | --- | --- |
| FR-MN-01 | System shall compute minimum delta-v avoidance maneuver for RED conjunctions with in-track prograde direction. | MUST | Done |
| FR-MN-02 | System shall calculate optimal maneuver lead time (hours before TCA). | MUST | Done |
| FR-MN-03 | System shall estimate fuel cost based on delta-v magnitude. | SHOULD | Done |
| FR-MN-04 | System shall perform cascade analysis: evaluate post-maneuver trajectory for secondary conjunction risks within 72h window. | SHOULD | Planned |
| FR-MN-05 | System shall optimize maneuver selection by balancing fuel efficiency against secondary collision risk (Pareto optimization). | SHOULD | Planned |

## 3.3 Data Management

| ID | Requirement | Priority | Status |
| --- | --- | --- | --- |
| FR-DM-01 | System shall persist all conjunction events to PostgreSQL with full CDM metadata. | MUST | Done |
| FR-DM-02 | System shall provide paginated conjunction history via REST API (GET /history). | MUST | Done |
| FR-DM-03 | System shall integrate Celestrak GP API for real-time TLE catalog data (stations, Starlink, active, Iridium, OneWeb, GEO). | MUST | Done |
| FR-DM-04 | System shall deduplicate CDM records by cdm_id to prevent duplicate storage. | SHOULD | Done |

## 3.4 User Interface

| ID | Requirement | Priority | Status |
| --- | --- | --- | --- |
| FR-UI-01 | Dashboard shall provide 3D orbital visualization using Three.js with interactive camera controls. | MUST | Done |
| FR-UI-02 | Dashboard shall display conjunction detail view with risk assessment, closest approach data, and maneuver recommendations. | MUST | Done |
| FR-UI-03 | Dashboard shall render Pc trend chart (Chart.js) with logarithmic scale, RED/YELLOW threshold lines, and miss distance overlay. | SHOULD | Done |
| FR-UI-04 | Dashboard shall display human-readable Pc values (percentage format + odds ratio) instead of scientific notation. | SHOULD | Done |
| FR-UI-05 | Dashboard shall support responsive layout for mobile devices (viewport < 768px). | SHOULD | Done |
| FR-UI-06 | Operator Portal shall provide login/register, API key management, conjunction history, and API documentation. | MUST | Done |

## 3.5 Authentication and Security

| ID | Requirement | Priority | Status |
| --- | --- | --- | --- |
| FR-AU-01 | System shall support JWT token-based authentication with 24h expiry. | MUST | Done |
| FR-AU-02 | System shall support API Key authentication for programmatic access. | MUST | Done |
| FR-AU-03 | System shall hash passwords using SHA-256 before storage. | MUST | Done |
| FR-AU-04 | System shall enforce unique email constraint on user registration. | MUST | Done |
| FR-AU-05 | System shall support API key regeneration with immediate invalidation of previous key. | SHOULD | Done |

## 3.6 Alerting and Notification

| ID | Requirement | Priority | Status |
| --- | --- | --- | --- |
| FR-AL-01 | System shall send email alerts for RED conjunction events via SMTP (TLS). | MUST | Done |
| FR-AL-02 | Email alerts shall include satellite names, miss distance, Pc, TCA, CDM ID, and maneuver recommendation. | MUST | Done |
| FR-AL-03 | System shall deduplicate alerts to prevent repeated notifications for the same conjunction event. | MUST | Done |
| FR-AL-04 | System shall support configurable alert recipients via environment variable. | SHOULD | Done |

# 4. Non-Functional Requirements

## 4.1 Performance

| ID | Requirement | Priority | Status |
| --- | --- | --- | --- |
| NFR-PF-01 | TLE analysis of 50 satellite pairs shall complete within 60 seconds. | MUST | Done |
| NFR-PF-02 | CDM fetch from Space-Track shall complete within 30 seconds. | MUST | Done |
| NFR-PF-03 | Engine memory footprint shall remain below 50 MB during normal operation. | SHOULD | Done |

## 4.2 Accuracy

| ID | Requirement | Priority | Status |
| --- | --- | --- | --- |
| NFR-AC-01 | CAS Pc calculation shall achieve >= 99% magnitude-order agreement with Space-Track CDM reference values. | MUST | Done |
| NFR-AC-02 | Risk classification (RED/YELLOW/GREEN) shall achieve >= 99% agreement with Space-Track classification. | MUST | Done |

Validation Results (SP-1): 5,202 CDM records validated. Magnitude-order accuracy: 99.9%. Risk classification accuracy: 99.9%. Sigma calibrated from 300m to 100m based on validation findings.

## 4.3 Availability and Monitoring

| ID | Requirement | Priority | Status |
| --- | --- | --- | --- |
| NFR-AV-01 | System shall expose /health endpoint for uptime monitoring. | MUST | Done |
| NFR-AV-02 | System shall expose /metrics endpoint in Prometheus-compatible format. | SHOULD | Done |
| NFR-AV-03 | System shall provide HTML status dashboard with auto-refresh at /status. | SHOULD | Done |

## 4.4 Data Sovereignty

| ID | Requirement | Priority | Status |
| --- | --- | --- | --- |
| NFR-DS-01 | All data processing and storage shall occur within EU-hosted infrastructure. | MUST | Done |
| NFR-DS-02 | System shall comply with GDPR requirements for personal data handling. | MUST | Done |
| NFR-DS-03 | System shall not depend on US-controlled infrastructure for critical safety computations. | SHOULD | Done |

# 5. REST API Specification

Base URL: https://casplatform.com/api

| Method | Endpoint | Description | Auth |
| --- | --- | --- | --- |
| GET | /health | Engine health check and version info | None |
| GET | /status | HTML status dashboard with metrics | None |
| GET | /metrics | Prometheus-format metrics endpoint | None |
| GET | /history?limit=N | Paginated conjunction event history | None |
| POST | /analyze | Analyze TLE data for conjunctions | None |
| POST | /spacetrack | Fetch CDM data from Space-Track | None |
| POST | /auth/register | Create operator account | None |
| POST | /auth/login | Login, returns JWT token + API key | None |
| GET | /auth/me | Get authenticated user info | Bearer / ApiKey |
| POST | /auth/regenerate-key | Regenerate API key | Bearer |

# 6. Validation and Verification

CAS has undergone systematic validation as part of Sprint SP-1 (Validation Engine). The validation compares CAS engine Pc calculations against Space-Track CDM reference values across the full dataset of 5,202 conjunction events.

## 6.1 Validation Results Summary

| Metric | Result |
| --- | --- |
| Total CDM records analyzed | 5,202 |
| Magnitude-order accuracy (+/- 1 order) | 99.9% (5,195 / 5,202) |
| Risk classification accuracy (RED/YELLOW/GREEN) | 99.9% (5,195 / 5,202) |
| Mean log10(CAS_Pc / ST_Pc) | -0.2376 (0 = perfect match) |
| Standard deviation | 0.2784 |
| Calibrated sigma | 100m (from 300m default) |

# 7. TRL Traceability Matrix

This section maps CAS capabilities to ESA ISO 16290 TRL criteria for software systems.

| TRL | ISO 16290 Criterion | CAS Evidence | Status |
| --- | --- | --- | --- |
| TRL 4 | Breadboard validation in laboratory environment | Dashboard + Engine working | ACHIEVED |
| TRL 5 | BETA version: all functions implemented | All FR- requirements Done | ACHIEVED |
| TRL 5 | Requirements frozen | This SRS document | ACHIEVED |
| TRL 5 | Full documentation | SRS + API Docs + Portal | ACHIEVED |
| TRL 5 | Domain of reuse defined | LEO/MEO/GEO (Section 2.3) | ACHIEVED |
| TRL 5 | Validated against requirements | 99.9% Pc accuracy (SP-1) | ACHIEVED |
| TRL 5 | Pilot users in relevant environment | Operator Portal live | IN PROGRESS |

# Appendix A: Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-03-27 | Mustafa Firinci | Initial release. All SP-1 through SP-4 requirements documented. |