CAS PLATFORM

Collision Avoidance System

SOFTWARE REQUIREMENTS SPECIFICATION

SRS v2.0

ECSS-E-ST-40C Uyumlu

April 2026 · Engine v0.7 · TRL 5

casplatform.com

CONFIDENTIAL

# Document information

| Document title | CAS Software Requirements Specification |
| --- | --- |
| Document no | CAS-SRS-002 |
| Version | 2.0 |
| Date | April 2026 |
| Author | Mustafa Fırıncı — Founder & Technical Lead |
| Standard | ECSS-E-ST-40C (Software Engineering) |
| TRL Seviyesi | TRL 5 (ESA ISO 16290) |
| Engine Versionu | v0.7 |
| Classification | Confidential |

## Revision history

| Version | Date | Changes |
| --- | --- | --- |
| 1.0 | Mart 2026 | Initial release — SP1-SP4 features, Pc calculation, CDM integration, watchlist, cascade v1 |
| 2.0 | April 2026 | Decision Engine v0.7, 72h Trend Analysis, Alert Funnel, Admin Panel, Tier Enforcement, Portal updates, 3D Catalog enrichment, theme support added |

# 1. Scope and purpose

## 1.1 System purpose

CAS (Collision Avoidance System) is a Conjunction Decision Support platform for satellite operators in Low Earth Orbit (LEO, 200-2000 km). The system automatically ingests Space-Track CDM data, performs independent Pc (Collision Probability) calculations, generates clear operator-language decisions for each conjunction event, and screens maneuver recommendations against secondary collision risk via cascade analysis.

## 1.2 Scope

This SRS covers CAS Platform Engine v0.7. The following subsystems are included:

(a) Data Katmanı: Space-Track CDM entegrasyonu, Celestrak TLE proxy, PostgreSQL veritabanı

(b) Analiz Katmanı: RK4+J2 propagation, Foster/Chan Pc, DecisionEngine, TrendAnalyzer, CascadeEngine

(c) Platform Katmanı: HTTP API sunucusu, JWT/API key auth, background scanner, email alert

(d) Kullanıcı Katmanı: Operator Portal, 3D Catalog, REST API, Admin Panel

## 1.3 Out of scope

GEO orbit analysis, autonomous maneuver execution, independent sensor data generation, ML-based prediction layers (TRL 6+ target).

# 2. General system description

## 2.1 System architecture

CAS operates on a single-server monolithic architecture. The Python HTTP engine (BaseHTTPRequestHandler) listens on port 8765 and is managed by systemd. Nginx reverse proxy handles HTTPS termination. Cloudflare CDN provides static file distribution.

Server: EU data centers

Database: PostgreSQL — conjunction_events, users, watchlist, watchlist_results, decision_results, admin_log, notification_prefs

Frontend: Statik HTML/CSS/JS — portal.html, index.html, catalog.html, landing.html

External dependencies: Space-Track API (CDM), Celestrak (TLE), CesiumJS (3D globe), SMTP (email)

## 2.2 User roles

| Role | Permissions |
| --- | --- |
| admin | All operator permissions + Admin Panel access (user CRUD, tier changes, audit log) |
| operator | Watchlist management, conjunction scanning, Decision Engine, Trend analysis, Cascade analysis, API access |
| viewer | Read-only access — dashboard and conjunction history viewing |

## 2.3 Tier definitions

| Tier | Price | Satellites | Data | Cascade | API |
| --- | --- | --- | --- | --- | --- |
| Free | $0/ay | 1 | 12h delayed | — | — |
| Starter | $499/ay | 3 | Real-time | Limited | Yes |
| Pro | $999/ay | 10-15 | Real-time | Full | Unlimited |
| Enterprise | Custom | Unlimited | Real-time | Full | Unlimited |

# 3. Functional requirements

## 3.1 Data Katmanı

| ID | Requirement | Description / Acceptance criteria |
| --- | --- | --- |
| FR-D-001 | Space-Track CDM ingestion | System shall perform hourly automatic CDM ingestion from basicspacedata/query/class/cdm_public endpoint. Rate limit: 1 full pull per 8 hours, 1 per hour for specific events. |
| FR-D-002 | CDM parsing and storage | Each CDM record (cdm_id, sat1, sat2, norad1, norad2, tca, pc, miss_distance, relative_velocity) shall be stored in PostgreSQL. Deduplication shall be performed via cdm_id. |
| FR-D-003 | Celestrak TLE proxy | /tle/<group> endpoint shall fetch TLE data from Celestrak and serve in JSON format. |
| FR-D-004 | Datatabanı indexleri | conjunction_events table shall have a composite index on (norad1, norad2, tca). |

## 3.2 Pc calculation engine

| ID | Requirement | Description / Acceptance criteria |
| --- | --- | --- |
| FR-P-001 | Foster/Chan 2D Pc | collision_probability() function shall implement Foster/Chan 2D Gaussian method. Sigma is currently fixed at 100m (covariance integration is TRL 6 target). |
| FR-P-002 | RK4+J2 propagation | Orbit propagation shall use 4th-order Runge-Kutta + J2 perturbation. |
| FR-P-003 | Risk classification | Pc ≥ 1e-4 = RED, Pc ≥ 1e-5 = YELLOW, Pc < 1e-5 = GREEN. risk_level() function shall enforce these thresholds. |
| FR-P-004 | Pc validation | CAS Pc values shall demonstrate ≥99% consistency with Space-Track CDM reference values. Current validation: 99.9% across 8,879 CDMs. |
| FR-P-005 | Human-readable Pc | All Pc values shall be displayed in '1 in 250' format. _format_pc_human() and frontend formatPc() functions shall produce this format. |

## 3.3 Decision Engine

| ID | Requirement | Description / Acceptance criteria |
| --- | --- | --- |
| FR-DE-001 | Automatic decision generation | System shall automatically generate recommendation, priority, confidence, and maneuver_summary for each conjunction event. |
| FR-DE-002 | Recommendation | 3 levels: 'Maneuver advised' (RED risk or Pc ≥ 1e-4), 'Monitor' (YELLOW or TCA passed), 'No action' (GREEN). If TCA has passed, even RED shall be downgraded to 'Monitor'. |
| FR-DE-003 | Priority scoring | 0-100 score: Pc (0-40) + miss_distance (0-30) + TCA proximity (0-30). Score ≥60 = HIGH, ≥30 = MEDIUM, <30 = LOW. |
| FR-DE-004 | Confidence | 3 levels: HIGH (TCA <12h and Pc very high/low), MEDIUM (TCA 12-48h), LOW (TCA >48h or uncertain). |
| FR-DE-005 | Maneuver summary | Operator language: 'Raise orbit ~Xm within Yh. Collision probability Z exceeds threshold.' format. Delta-v, altitude change, and time window shall be computed automatically. |
| FR-DE-006 | Alert funnel | alert_summary: {total, review, critical} fields shall be computed. total=all CDM, review=YELLOW+RED, critical=RED only. |
| FR-DE-007 | Decision storage | Shall be stored in decision_results DB table: norad_id, recommendation, priority, confidence, maneuver_summary, max_pc, min_miss_m, timestamp. |
| FR-DE-008 | Decision API | GET /api/decision — latest decisions. GET /api/decision/dashboard — aggregate summary. GET /api/decision/scan — fresh scan + decision. |

## 3.4 Trend analysis

| ID | Requirement | Description / Acceptance criteria |
| --- | --- | --- |
| FR-TR-001 | 72-hour historical analysis | compute_trend() shall analyze the last 72 hours of CDM data in 6-hour buckets to compute Pc trend. |
| FR-TR-002 | Trend direction | 3 levels: 'increasing' (last 3 bucket average > previous 3), 'decreasing' (opposite), 'stable'. |
| FR-TR-003 | 72h forecast | _simple_forecast() shall extrapolate current trend rate to produce risk direction forecast: 'escalating', 'de-escalating', 'stable'. |
| FR-TR-004 | Trend API | GET /api/trend/<norad> — per-satellite trend. GET /api/trend/forecast — all watchlist forecast. |

## 3.5 Cascade analysis

| ID | Requirement | Description / Acceptance criteria |
| --- | --- | --- |
| FR-CA-001 | Maneuver candidate generation | generate_maneuver_candidates() shall produce 12 candidates: 4 directions (prograde, retrograde, normal, anti-normal) x 3 delta-v (0.1, 0.5, 1.0 m/s). |
| FR-CA-002 | Catalog screening | fetch_catalog_tles() shall fetch all objects in the target altitude band (±100 km) from Celestrak. Expected count: 10,000+. |
| FR-CA-003 | Secondary risk screening | Post-maneuver orbit shall be propagated for each candidate and screened against catalog objects. <10 km approach = secondary risk. |
| FR-CA-004 | Pareto optimization | select_optimal_maneuver_v2() shall select the lowest delta-v candidate among those with zero secondary risk. |
| FR-CA-005 | Cascade API | POST /cascade — body: {norad_id, tle_line1, tle_line2, target_norad}. Response: candidate list, optimal selection, secondary risk report. |
| FR-CA-006 | Performance | Cascade analysis shall complete in ~10 seconds (parallel screening via multiprocessing). |

## 3.6 Watchlist ve Satellites Yönetimi

| ID | Requirement | Description / Acceptance criteria |
| --- | --- | --- |
| FR-WL-001 | Satellites ekleme | POST /watchlist/add — norad_id ve sat_name zorunlu. TLE Line 1/2 opsiyonel. Tier uydu limitine uyulmalıdır. |
| FR-WL-002 | Satellites silme | POST /watchlist/remove — remove from watchlist by norad_id. |
| FR-WL-003 | Watchlist listing | GET /watchlist — return all user satellites. |
| FR-WL-004 | Automatic scanning | Background scanner shall scan all watchlist satellites hourly in parallel (ThreadPoolExecutor). |
| FR-WL-005 | Tier enforcement | When adding satellites, a limit check should be performed based on the user's tier. Exceeding the limit will result in a 403 + upgrade message. |

## 3.7 Admin panel

| ID | Requirement | Description / Acceptance criteria |
| --- | --- | --- |
| FR-AD-001 | User CRUD | POST /admin/user/create, /update, /delete, /set-tier, /toggle — requires admin role. |
| FR-AD-002 | System statistics | GET /admin/stats — total users, active users, tier distribution, total watchlist, total CDMs. |
| FR-AD-003 | Audit log | All admin actions shall be logged to admin_log DB table: action, target_user_id, details, admin_user_id, timestamp. |
| FR-AD-004 | User list | GET /admin/users — all users + tier + watchlist count + last login. |

## 3.8 Authentication

| ID | Requirement | Description / Acceptance criteria |
| --- | --- | --- |
| FR-AU-001 | JWT token | POST /auth/login — email + password → JWT token (24h TTL). Authorization: Bearer <token>. |
| FR-AU-002 | API key | Each user is assigned an API key on registration. Authorization: ApiKey <key>. Regenerate via POST /auth/regenerate-key. |
| FR-AU-003 | Registration | POST /auth/register — email, password, name. Otomatik API key üretimi. Yessayılan tier: free. |

## 3.9 Email alerts

| ID | Requirement | Description / Acceptance criteria |
| --- | --- | --- |
| FR-EM-001 | Alert triggering | When background scanner detects RED or YELLOW risk, email shall be sent to the relevant watchlist owner. |
| FR-EM-002 | Watchlist filter | Emails shall only be sent for satellites in the user's watchlist. |
| FR-EM-003 | SMTP | Email SMTP (mustafa@casplatform.com). |

# 4. Fonksiyonel Olmayan Requirementler

| ID | Requirement | Description / Acceptance criteria |
| --- | --- | --- |
| NFR-001 | Uptime | System shall operate 24/7. Automatic restart via systemd watchdog. |
| NFR-002 | API response time | Average <500ms. Cascade analysis <15 seconds. |
| NFR-003 | Data tutarlılığı | Pc calculation ≥99% consistency with Space-Track CDM. |
| NFR-004 | Security | HTTPS (Cloudflare), JWT token (24h TTL), API key auth, bcrypt password hashing, UFW + fail2ban. |
| NFR-005 | GDPR | EU infrastructure, data minimization, user deletion right (via admin panel). |
| NFR-006 | Scalability | Current architecture shall support 100+ users and 500+ watchlist satellites. |
| NFR-007 | Logging | All API calls and error conditions shall be written to server logs. |
| NFR-008 | Theme support | Portal and landing page shall support dark/light theme toggle. Preference shall be persisted in localStorage. |

# 5. API endpoint reference

All endpoints are accessible at https://casplatform.com. Endpoints requiring authentication (🔒) require an Authorization header.

| Method | Endpoint | Auth | Description |
| --- | --- | --- | --- |
| GET | /health | — | System status + feature list |
| POST | /auth/login | — | Get JWT token |
| POST | /auth/register | — | Create account |
| GET | /auth/me | 🔒 JWT/API | User info |
| POST | /auth/regenerate-key | 🔒 JWT | Regenerate API key |
| GET | /watchlist | 🔒 JWT/API | List watchlist |
| POST | /watchlist/add | 🔒 JWT/API | Satellites ekle (tier limit kontrolü) |
| POST | /watchlist/remove | 🔒 JWT/API | Satellites sil |
| GET | /watchlist/scan | 🔒 JWT/API | Scan all watchlist satellites |
| GET | /api/decision | 🔒 JWT/API | Latest decisions |
| GET | /api/decision/dashboard | 🔒 JWT/API | Aggregate decision summary + alert funnel |
| GET | /api/decision/scan | 🔒 JWT/API | Fresh scan + compute decisions |
| GET | /api/trend/<norad> | 🔒 JWT/API | 72h Pc trend analysis |
| GET | /api/trend/forecast | 🔒 JWT/API | All watchlist risk forecast |
| POST | /cascade | 🔒 JWT/API | Run cascade analysis |
| POST | /spacetrack/auto | — | Fetch Space-Track CDMs |
| GET | /api/history | — | Conjunction history |
| GET | /admin/* | 🔒 Admin | Admin panel endpoints (9 total) |

# 6. Known limitations and future work

## 6.1 Known limitations

(a) Covariance matrix: Space-Track cdm_public class does not include covariance data (CR_R, CT_T, CN_N). Fixed σ=100m calibration value is used. Organizational Account (ODR) application has been submitted.

(b) LEO focus: System currently supports LEO (200-2000 km) orbits only. GEO support is TRL 6+ target.

(c) Forecast limitation: 72h forecast uses simple linear extrapolation. Statistical confidence is limited.

(d) Confidence: Without covariance, confidence calculation is a rough approximation. Quantitative confidence will come with TRL 6.

## 6.2 TRL 6 targets

(a) Space-Track expandedspacedata access — real covariance integration

(b) ML Layer 1: False positive reduction (NASA 2M+ CDM dataset)

(c) Pilot user operational testing

(d) Independent performance validation report

(e) Shadow mode: comparison of real operator decisions with CAS decisions

CAS Platform · casplatform.com · mustafa@casplatform.com · SRS v2.0 · April 2026