CAS PLATFORM

CONJUNCTION DECISION SUPPORT

VERIFICATION CROSS-REFERENCE MATRIX

Verification Cross-Reference Matrix

CAS Platform · Independent European Conjunction Decision Support

| VERSION 2.2 | DATE August 2026 | MATURITY TRL 5 | STATUS PUBLIC |
| --- | --- | --- | --- |

Document ID  CAS-VCRM-002    Standard  ECSS-E-ST-40C

casplatform.com

# Document Information

| Field | Value |
| --- | --- |
| Document Title | CAS Platform — Verification Cross-Reference Matrix |
| Document ID | CAS-VCRM-002 |
| Version | 2.2 |
| Date | August 2026 |
| Traces To | CAS-SRS-004 (Software Requirements Specification v4.2) |
| Standard | ECSS-E-ST-40C (Software Engineering) |
| Maturity Level | TRL 5 (ESA ISO 16290) |
| Classification | Public |

This matrix traces every requirement in SRS v4.2 to its verification method and evidence. All 115 requirements are verified. This revision adds the data-layer corrections made after the July retrieval-policy incident, the Data Health architecture, the Orbital Collision-Burden Index and portal support.92 of 93 requirements are verified; 1 (password reset) is planned for general availability.

# 1. Verification Summary

| Category | Total | Verified | Status |
| --- | --- | --- | --- |
| Data Layer (incl. canonical CDM) | 10 | 10/10 | PASS |
| Probability of Collision | 5 | 5/5 | PASS |
| Decision Engine | 9 | 9/9 | PASS |
| Trend Analysis | 5 | 5/5 | PASS |
| Cascade Analysis | 6 | 6/6 | PASS |
| VLEO Drag-Aware Layer | 9 | 9/9 | PASS |
| ML Layer 1 | 5 | 5/5 | PASS |
| Real-Time / Launch / Mission | 4 | 4/4 | PASS |
| Watchlist | 6 | 6/6 | PASS |
| Admin | 4 | 4/4 | PASS |
| Authentication | 5 | 5/5 | PASS |
| Supporting Services (EU SST, SWPC, Email, Contact, Historical) | 20 | 20/20 | PASS |
| Data Health | 6 | 6/6 | PASS |
| Orbital Collision-Burden Index | 8 | 8/8 | PASS |
| Portal Support | 3 | 3/3 | PASS |
| Non-Functional | 10 | 10/10 | PASS |
| TOTAL | 115 | 115/115 | 100% VERIFIED |

VERIFICATION METHODS Test = automated pytest (134 unit + 135 integration + smoke). Ops = operational evidence from the live system. Analysis = validation report. Inspection = manual review.

# 2. Detailed Verification Matrix

## 2.1 Data Layer

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-D-001 | Space-Track CDM ingestion | Test + Ops | test_cdm_parser.py (22 tests); hourly cron active 90+ days | PASS |
| FR-D-002 | CDM persistence with dedup | Test + Ops | UPSERT on cdm_id; 20,899 events, 333 unique CDM IDs in DB | PASS |
| FR-D-003 | Celestrak TLE proxy | Ops | /tle/ live; 9 Celestrak groups cached (6h TTL) | PASS |
| FR-D-004 | Multi-source catalog ingestion | Ops | 30,884+ objects (DEBRIS/ROCKET BODY/PAYLOAD/UNKNOWN); dedup on NORAD | PASS |
| FR-D-005 | Database indexes | Inspection | Composite index (norad1,norad2,tca); unique on cdm_id verified | PASS |
| FR-D-006 | Canonical CDM schema | Test + Inspection | schemas/ 6 files, 135 fields, 9 enums; mappers report coverage; kelvins 10K rows error-free | PASS |
| FR-D-007 | Incremental CDM retrieval | Test + Ops | Query restricted to messages created in the preceding day, ordered by creation time, limit 200, minimum-probability filter applied | PASS |
| FR-D-008 | Scheduling and failover | Test + Ops | Three retrievals per day; backup credential tried sequentially only after primary login failure | PASS |
| FR-D-009 | Catalogue overwrite guard | Test | Zero-object result leaves cache untouched, exits non-zero and records failure; directory rebuild available below viable count | PASS |
| FR-D-010 | Conjunction de-duplication | Test | Latest state selected per conjunction identifier in analytics and reporting queries | PASS |

## 2.2 Probability of Collision

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-P-001 | Foster/Chan 2D Pc | Test | test_collision_probability.py (18 tests); Bessel I₀, N=200 integration | PASS |
| FR-P-002 | Orbital propagation | Test + Ops | RK4 engine propagator; vleo.py RK4+drag propagator (50 tests) | PASS |
| FR-P-003 | Risk classification | Test | test_risk_level.py (9 tests); RED/YELLOW/GREEN thresholds | PASS |
| FR-P-004 | Pc consistency validation | Analysis | Validation Report: 99.9% magnitude-order consistency (5,202 CDMs) | PASS |
| FR-P-005 | Human-readable Pc format | Inspection | '1 in N' + scientific notation with threshold context in portal | PASS |

## 2.3 Decision Engine

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-DE-001 | Automated decision generation | Test | test_decision_engine.py (15 tests); auto per conjunction | PASS |
| FR-DE-002 | Recommendation taxonomy | Test | Maneuver advised / Monitor / No action; TCA-elapsed downgrade | PASS |
| FR-DE-003 | Priority scoring | Test | Multi-factor (Pc, miss, TCA) 0–100; HIGH/MEDIUM/LOW | PASS |
| FR-DE-004 | Confidence rating | Test | HIGH/MEDIUM/LOW confidence output | PASS |
| FR-DE-005 | Maneuver summary | Test + Ops | test_compute_dv.py (12 tests); ΔV, altitude, window in portal | PASS |
| FR-DE-006 | Alert funnel | Ops | total→review→critical compression demonstrated in portal | PASS |
| FR-DE-007 | Decision persistence | Ops | decision_results stored with timestamps | PASS |
| FR-DE-008 | Decision API | Ops | /api/decision, /dashboard, /scan live | PASS |
| FR-DE-009 | Operator decision recording | Ops | Approve/monitor actions logged with timestamp + user ID | PASS |

## 2.4 Trend Analysis

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-TR-001 | Trend computation | Ops | compute_trend(); 72h sliding window, 6h buckets, regression slope | PASS |
| FR-TR-002 | Trend classification | Ops | increasing / decreasing / stable labels | PASS |
| FR-TR-003 | Risk forecast | Ops | 72h forward forecast with confidence band in portal | PASS |
| FR-TR-004 | Trend API | Ops | /api/trend/<norad> and /api/trend/forecast live | PASS |
| FR-TR-005 | Insufficient data flag | Ops | Returns insufficient_data when <3 points in window | PASS |

## 2.5 Cascade Analysis

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-CA-001 | Maneuver candidate generation | Test | 12 candidates: 4 directions × 3 magnitudes | PASS |
| FR-CA-002 | Catalog screening | Test + Ops | test_rank_debris.py (8 tests); 10K+ objects per maneuver | PASS |
| FR-CA-003 | Secondary risk evaluation | Test | Post-maneuver conjunction check; <10 km flagged | PASS |
| FR-CA-004 | Pareto optimization | Ops | Minimum ΔV + zero secondary risk selection | PASS |
| FR-CA-005 | Cascade API | Ops | POST /cascade; candidates + optimal + secondary-risk report | PASS |
| FR-CA-006 | Performance | Ops | Full cascade completes within target on production hardware | PASS |

## 2.6 VLEO Drag-Aware Layer

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-VL-001 | Regime detection | Test + Ops | detect_regime(); VLEO<400 / hybrid<450; verified on EAGLEEYE (356.8 km) | PASS |
| FR-VL-002 | Atmospheric density | Test + Ops | NRLMSIS 2.1 (pymsis) with USSA76 fallback; model-tagged in every result | PASS |
| FR-VL-003 | Drag-aware propagation | Test | RK4 Cowell + drag; decay and σ-inflation estimates (50 tests) | PASS |
| FR-VL-004 | Drag make-up ΔV forecast | Test + Ops | drag_delta_v_forecast(); daily + horizon ΔV + unmaintained decay | PASS |
| FR-VL-005 | Urgency triage signal | Test + Ops | vleo_urgency_score() 0–100; verified 63.2 on EAGLEEYE scenario | PASS |
| FR-VL-006 | Station-keeping simulation | Test + Ops | simulate_continuous_maneuver(); full-hold net ~0, ΔV 11.35 m/s, 2.19 mN @500 kg | PASS |
| FR-VL-007 | Fragmentation cascade | Test + Ops | assess_vleo_cascade(); NASA SBM; self-cleaning 356 km 8 d vs 800 km 26 yr | PASS |
| FR-VL-008 | Fuel trade-space | Test + Ops | vleo_fuel_tradespace(); CW ΔV~Δsep/(3t); ≤30% prograde reuse | PASS |
| FR-VL-009 | VLEO API | Ops | 6 endpoints live (status/assess/delta-v/maneuver-sim/cascade/fuel-tradespace); 401/422 validation verified | PASS |

## 2.7 ML Layer 1

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-ML-001 | Model (XGBoost) | Test + Analysis | xgb_layer1_production.json; AUC 0.953 held-out; 7 leakage features removed | PASS |
| FR-ML-002 | Calibrated tiers | Analysis | RED ≥0.137 / YELLOW ≥0.051 / GREEN <0.051; calibration-derived | PASS |
| FR-ML-003 | Explainability | Test + Ops | SHAP TreeExplainer (shap 0.52.0) bound at inference; per-event attributions | PASS |
| FR-ML-004 | Coverage gate / fail-safe | Test | Sparse CDM → UNAVAILABLE tier; defers to deterministic Pc funnel | PASS |
| FR-ML-005 | Decision-support boundary | Inspection | ML output augments, never overrides, deterministic Pc + operator | PASS |

## 2.8 Real-Time / Launch / Mission

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-RT-001 | WebSocket conjunction stream | Ops | WebSocket channel live; heartbeat + reconnect; Nginx upgrade map + 3600s timeout | PASS |
| FR-RT-002 | Launch screening | Ops | TheSpaceDevs feed; launch-window conjunction exposure surfaced | PASS |
| FR-RT-003 | Mission design tools | Ops | Launch schedule context + maneuver/lead-time trade-spaces in portal | PASS |
| FR-RT-004 | Multi-channel notifications | Ops | notification_prefs; multi-channel alert dispatch | PASS |

## 2.9 Watchlist

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-WL-001 | Add satellite | Ops | POST /watchlist/add; auto altitude + regime detection | PASS |
| FR-WL-002 | Remove satellite | Ops | POST /watchlist/remove | PASS |
| FR-WL-003 | List watchlist | Ops | GET /watchlist; regime badge (LEO/VLEO/HYBRID) | PASS |
| FR-WL-004 | Automated scanning | Ops | Hourly scan updates altitude/regime; RED triggers email | PASS |
| FR-WL-005 | Tier enforcement | Ops | Free=1, Starter=5, Pro=20, Enterprise=unlimited; HTTP 403 over limit | PASS |
| FR-WL-006 | Watchlist-scoped history | Test + Ops | Admin global / operator watchlist-scoped / no-auth fail-closed (COSMOS leak fixed) | PASS |

## 2.10 Admin

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-AD-001 | User CRUD | Ops | Create / update tier / delete (cascade-protected) / toggle | PASS |
| FR-AD-002 | System statistics | Ops | User count, tier distribution, watchlist + CDM totals | PASS |
| FR-AD-003 | Audit log | Ops | admin_log with action, target, details, UA, client IP, timestamp | PASS |
| FR-AD-004 | User listing | Ops | User table with tier, watchlist count, last login | PASS |

## 2.11 Authentication

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-AU-001 | JWT issuance | Ops | HMAC-SHA256 3-segment JWT (24h); legacy fallback during sunset | PASS |
| FR-AU-002 | API key authentication | Ops | cas_<32-hex> key; Authorization: ApiKey; regenerate endpoint | PASS |
| FR-AU-003 | Registration with email verification | Ops | register → verification email → confirm → active | PASS |
| FR-AU-004 | Verification email deliverability | Ops | Namecheap SMTP; SPF/DKIM; multipart HTML; tested delivery | PASS |
| FR-AU-005 | Password reset | Test | POST /auth/forgot-password issues a single-use token with a one-hour validity window, stored server-side and consumed on reset; the reset link is delivered by email. | PASS |

## 2.12 Supporting Services

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-EU-001 | EU SST OAuth2 client | Ops | Keycloak OAuth2 token refresh; eusst_sync.py operational | PASS |
| FR-EU-002 | Re-entry feed sync | Ops | 800+ RE events; 6h cron; eusst_re_events | PASS |
| FR-EU-003 | Fragmentation feed sync | Ops | 50+ FG events; featured-event highlighting | PASS |
| FR-EU-004 | Incremental sync state | Ops | eusst_sync_state tracks last sync per service | PASS |
| FR-EU-005 | Tier-aware presentation | Ops | Aggregate (all tiers); detail tabs (Starter+); ToS §5.1 | PASS |
| FR-EU-006 | Portal UI | Ops | Sidebar accordion: Aggregate / Reentries / Fragmentations | PASS |
| FR-SW-001 | NOAA SWPC ingestion | Ops | space_weather_sync.py; hourly Kp, F10.7, X-ray, alerts | PASS |
| FR-SW-002 | Storage | Ops | spaceweather_history; 7-day retrieval | PASS |
| FR-SW-003 | Operational impact display | Ops | Kp/F10.7/X-ray panel with operator-language interpretation | PASS |
| FR-SW-004 | Overview integration | Ops | Space-weather summary card on Overview | PASS |
| FR-SW-005 | Space-weather API | Ops | /api/spaceweather/current and /history live | PASS |
| FR-EM-001 | Alert trigger | Ops | RED/YELLOW conjunction triggers email to owner | PASS |
| FR-EM-002 | Watchlist filter | Ops | Only alerts for user's own watchlist | PASS |
| FR-EM-003 | Configurable threshold | Ops | Notification preferences in Settings | PASS |
| FR-EM-004 | SMTP transport | Ops | Namecheap Private Email; multipart MIME | PASS |
| FR-CT-001 | Contact submission endpoint | Ops | POST /api/contact; contact_submissions | PASS |
| FR-CT-002 | Anti-abuse | Ops | Honeypot + 3/hr/IP rate limit + min-length | PASS |
| FR-CT-003 | Contact notification | Ops | SMTP notification on each submission | PASS |
| FR-HE-001 | Historical reference set | Ops | Real CDMs in Historical Events; threshold tooltip | PASS |
| FR-HE-002 | No synthetic data | Inspection | All entries from real conjunction records | PASS |

## 2.13 Non-Functional

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| NFR-001 | Uptime | Ops | 90+ days continuous; systemd auto-restart | PASS |
| NFR-002 | API response time | Ops | <500 ms read; cascade <15 s; VLEO sims single-digit s | PASS |
| NFR-003 | Pc consistency | Analysis | 99.9% internal recomputation consistency | PASS |
| NFR-004 | Security | Ops | HMAC-SHA256 JWT + API key; bcrypt; HTTPS only | PASS |
| NFR-005 | Network hardening | Ops | UFW CDN-only ingress; fail2ban on SSH | PASS |
| NFR-006 | GDPR compliance | Inspection | EU-hosted; data minimization; privacy policy published | PASS |
| NFR-007 | EU SST data handling | Inspection | §5.1 compliant; no raw payload exposed; disclaimer on endpoints | PASS |
| NFR-008 | Data sovereignty | Inspection | Derived analysis processed + stored within EU | PASS |
| NFR-009 | Model governance | Test + Inspection | Calibrated, SHAP-explainable, fail-safe ML; no autonomous decisions | PASS |
| NFR-010 | Theme & accessibility | Ops + Inspection | Dark/light; ARIA; keyboard nav; reduced-motion | PASS |

## 2.14 Data Health

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-DH-001 | Central health register | Test + Ops | Seven sources registered with expected intervals from 60 to 10080 minutes; last success, last attempt, status and failure count recorded | PASS |
| FR-DH-002 | Last-good-data preservation | Test | Empty upstream result skips the write and reports failure; previous data retained | PASS |
| FR-DH-003 | Staleness determination | Ops | Catalogue source detected stale at 12465 minutes against a 1440-minute interval; source restored and returned to current | PASS |
| FR-DH-004 | Failure and recovery notification | Test | Problem notification on first failure and recovery notification on first subsequent success both delivered end to end | PASS |
| FR-DH-005 | Status escalation | Test | Degraded below three consecutive failures, failed at three or more; last error retained | PASS |
| FR-DH-006 | Freshness surfaced | Test + Ops | Health endpoint returns all seven sources; portal banner names stale sources on entry and is hidden when all are current | PASS |

## 2.15 Orbital Collision-Burden Index

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-IN-001 | Kinetic burden imported | Inspection | Burden term imported from the production service; no local recomputation path exists | PASS |
| FR-IN-002 | Composite index | Test | Weights 1.0 / 0.5 / 0.5 applied and returned with every response | PASS |
| FR-IN-003 | Calibration and clipping | Test | Clipped value, raw value and clipping flag all returned; range [0.5, 2.0] | PASS |
| FR-IN-004 | Severity | Test | Nested thresholds 200/100/50/25 m with weights 1/2/4/8, normalised by total weight | PASS |
| FR-IN-005 | Cascade exposure | Test | Population within +/-50 km combined with 90 percent clearing time, log-normalised; substitution reported when clearing time unavailable | PASS |
| FR-IN-006 | Unavailable components declared | Test | Missing components omitted from the product and named in an explicit list; trend currently unavailable | PASS |
| FR-IN-007 | Relative index | Test | Fewer than three peer orbits rejected; percentile within peer set returned per result | PASS |
| FR-IN-008 | Access control and framing | Test | Insurer role required over bearer authentication; catalogue source, observation basis, confidence levels and is / is-not statement present in every response | PASS |

## 2.16 Portal Support

| ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-SP-001 | Ticket submission | Test | Category, subject and body accepted; portal, role and client context recorded automatically | PASS |
| FR-SP-002 | Screenshot attachment | Test | Non-PNG/JPEG data URI rejected; attachment above the 2 MB limit rejected with payload-too-large | PASS |
| FR-SP-003 | Submission rate limit | Test | Requests beyond the hourly per-user limit rejected with too-many-requests | PASS |

Method key: Test = automated pytest · Ops = operational evidence from the live system · Analysis = validation report · Inspection = manual review · Planned = scheduled for general availability.

— End of Matrix —